Trefwoorden

Stochastic subsoil schematization, soil profile, soil properties, macro stability, piping, flow slide

Samenvatting

This document contains the requirements and a global description of the functional design for the application D-Soil Model, which will be a part of the WTI 2017 software. The application supports the (stochastic) subsoil schematizations for the failure mechanism macro stability, piping and flow slide. A subsoil schematization for an area or segment consists of 1D/20 soil profiles and soil material properties.

| Versie | Datum | Auteur | Paraaf | Review | Paraaf | Goedkeuring | Paraaf |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | maa. 2016 | Irene van der Zwan |  | Kin Sun Lam |  | Maya Sule |  |

Status concept Dit document is een concept en uitsluitend bedoeld voor discussiedoeleinden. Aan de inhoud van dit rapport kunnen noch door de opdrachtgever, noch door derden rechten worden ontleend.

Inhoud

1	Introduction	1

1.1	Purpose and scope	1

1.2	Other system documents	1

2	Constraints	1

2.1	General constraints	1

2.2	Capabilities related to the failure mechanisms	1

2.3	Capabilities related to the interface	1

2.4	Items that are not supported	1

3	Requirements	1

3.1	General and non-functional requirements	1

3.2	Functional requirements	1

3.2.1	Following out of CNS 3	1

3.2.2	Following out of CNS 4	1

3.2.3	Following out of CNS 5	1

3.2.4	Following out of CNS 6	1

3.2.5	Following out of CNS 7	1

3.2.6	Following out of CNS 8	1

3.2.7	Following out of CNS 9	1

4	Functional design	1

4.1	Schematization proces	1

4.2	Use case 1 Collect relevant subsoil information	1

4.2.1	Import data	1

All data can be imported separately, but two items are depending on others (cannot exist without the other):	1

4.2.2	View data on map	1

4.2.3	View data in table	1

4.2.4	View data in a drawing (cross section).	1

4.3	Use case 2 User must be able to combine subsoil information and view the combination	1

4.3.1	Connect items in the cross section (one to one)	1

4.3.2	Connect items in batch (several to one)	1

4.3.3	View combination in a drawing.	1

4.4	Use case 3 User must be able to adept the WTI-SOS 2017 to local situation.	1

4.5	Make subsoil schematization for the failure mechanism Macrostabiliteit binnenwaarts (STBI)	1

4.6	Make subsoil schematization for the failure mechanism Piping (STPH)	1

4.7	Error handling	1

5	References	1

Bijlage(n)

A	A-1

# Introduction

## Purpose and scope

This document contains the requirements and the functional design for the application D-Soil Model, which will form a part of the WTI 2017 software applications. The application supports at least the (stochastic) subsoil schematizations for the failure mechanism macro stability and. A subsoil schematization for an area or segment consists of soil profiles and soil material properties.

D-Soil Model is a software tool for schematization of the subsoil and is developed for the assessment of primary dikes according to the WTI2017. In this process of assessment the schematization of a stochastic subsoil model (SOSWTI2017) is a major source of information. And the D-Soil Model development focusses on use of this stochastic subsoil model (SOSWTI2017).

The document will not give any background on the context of the WTI project and on the derivation or motivation of the supported schematizations. For this purpose the reader is referred to the VTV 2017 and to its supporting technical reports and their background reports underneath.

This document is a revision of 1209430-003 WTI 2017 D-Soil Model Requirements and Functional Design and is applicable to version 15.1. In chapter 2 the constraints are described. These are translated to requirements, chapter 3. Chapter 4 describes the functional design of D-Soil Model (actual version and not implemented functionality). The functional design describes the functionality via user cases; the answer to ‘what?’. The answer to ‘How?’ is given in the Technical Design.

A glossary of used terms is placed in appendix A.

## Other system documents

For the development of D-Soil Model different documents are available. The titles are posted in the table below.

| Title | Content |
| --- | --- |
| Requirements and functional design | This document |
| Technical Design [2] | Definition of the different software components and their interaction |
| Test plan and Test report [3] | Description of the different regression and acceptation tests, including target values. |

# Constraints

## General constraints

CNS 1. 	As a general constraint, the development process needs to comply with the the Deltares DQMS system.

CNS 2. 	As a general constraint, the application needs to comply with the relevant general requirements and further design rules for the programming, documentation and testing of WTI software. This set of requirements and rules are available in a separate document [1].

## Capabilities related to the failure mechanisms

CNS 3. 	Import data from existing data formats.

CNS 4.	D-Soil Model must provide all soil parameters needed for the stability- and piping calculations in the WTI software.

CNS 5.	The application is at least suitable to support the failure mechanisms Piping, Macro Stability and Flow Slide.

CNS 6.	The schematization of the subsoil for these mechanisms must be complete and the calculation software can use the subsoil schematizations directly.

CNS 7.	The application is applicable for stochastic subsoil modeling.

## Capabilities related to the interface

CNS 8.	The user must have control to view the building blocks of the subsoil schematization.

CNS 9.	The user must have control to edit the building blocks of the subsoil schematization.

## Items that are not supported

CNS 10.	No data related to water pressures are stored in the application. Only parameters that are directly related to the soil.

CNS 11.	Surface lines and characteristic points are only for reference and not suitable for calculation purposes in calculation software, they are only used for internal purposes.

# Requirements

This chapter describes the requirements that follow out of the constraints. Most requirements are too general to be programmed directly. Chapter 4 specifies the requirements further as a basis for the following technical design in user stories.

## General and non-functional requirements

The externally defined general and non-functional requirements for all WTI software apply;

see section 2.1 where CNS 1 and CNS 2 are defined.

## Functional requirements

### Following out of CNS 3

Data to import consists of data needed for WTI software and data to support the subsoil schematization by the user (such as polder water level, surface lines and characteristic points).

REQ 3 Format of import data.

The required import files are:

- sti file and mdb file from D-GeoStability; for subsoil schematizations

- (2D profiles) and strength parameters.

- csv files; description of the segments and 1Dsoil profiles.

- shape files; location of segments, 2D profiles, CPT's, borings.

### Following out of CNS 4

REQ 4 Soilparameters

All soil parameters needed for the stability- and piping calculations in the WTI software must be incorporated in D-Soil Model.

### Following out of CNS 5

REQ 5 The  application  gathers  the  (stochastic)  subsoil  schematizations  input  and parameters for the following kernels:

- WTI macro stability kernel.

- WTI piping kernel.

- WTI flow slide kernel.

D-Soil Model can save this output into one file.

### Following out of CNS 6

REQ 6 Schematization of subsoil

For each failure mechanism the subsoil is described in soil segments consisting of one of more scenario(s) of a soil profile with a probability of appearance. These soil profiles are described in either 1D or 20 layer schematization. A soil segment can have a combination of

1D and 20 soil profiles. Each layer is described with at least all the parameters needed for the relevant failure mechanism. This output is gathered in one file, which can be directly imported in WTI software.

### Following out of CNS 7

REQ 7 Stochastic schematization of subsoil.

All the parameters of above described schematization can consist as a stochastic variable or "as is" value (characteristic value or design value) or as both. D-Soil Model stores both values. The program using D-Soil Model projects can decide which one to use for which calculation; the two values are not connected to each other; changing one does not change the value of the other.

The stochastic variables can be stored as a mean value with a distribution type and the distribution itself. An overview of al variables is given in appendix X.

### Following out of CNS 8

The building blocks of the subsoil schematization are:

SOSWTI2017

- Soilsegments

- Location on map

- Failure mechanism

- Scenario of soil profiles with the probability of appearance

- Soilprofiles

- Profile data: yield stresses

- Layer data: material name, boundaries, aquifer/aquitard

- Soilmaterials

- Soilparameters per material name

Supporting data (optional)

- CPT

- Boring

- Lab data Hydraulic levels

REQ 8 View the building blocks of the subsoil schematization

The subsoil schematization must be visible to the user:

All data is shown in tables to provide an overview and edit possibilities.

Data with a geographical reference (segment, CPT, boring, etc.) is shown on a map (with legend and label-option).

Schematization of layers or other graphs (e.g. CPT) is visible as cross sections.

Link to file-related programs

Depending on the kind of file format, the files can also be viewed in a format-related program;

e.g. *.sti (2D profiles) in DGeoStability, *.gef (CPT and borings) in a GEF viewer.

### Following out of CNS 9

REQ 9 Edit and generate the building blocks of the subsoil schematization

The user is able to edit all the building blocks. The user is responsible for the edits, D-Soil Model gives validation messages when applicable (e.g. minimum and maximum values).

# Functional design

## Schematization proces

D-Soil Model is a software tool for schematization of the subsoil and is developed for the assessment of primary dikes according to the WTI 2017. In this process of assessment the schematization of a stochastic subsoil model (WTI-SOS 2017) is a major source of information. And the D-Soil Model development focusses on use of this stochastic subsoil model. The proces of subsoil schematization consists of the following steps:

- Collect relevant subsoil information

- Combine and compare these information

- Make subsoil schematization for macrostabiliteit

- Make subsoil schematization for piping

- Store the schematization

- Use schematization in other applications

The user must be able to make a local schematization of the WTI-SOS 2017 schematization and use this schematization for calculations of the failure mechanisms Macrostabiliteit and Piping.

## Use case 1 Collect relevant subsoil information

Possible information

  - WTI-SOS 2017

Content of WTI-SOS 2017:

Ondergrondsegmenten

Ondergrondprofielen met laagscheidingen (gemiddeld / minimum / maximum)

Lagen met materiaalnamen en eventuele opmerkingen

  - Grondonderzoek; sonderingen, boringen

  - Excisting geometries (from D-Geostability)

  - Geotechnisch lengteprofiel

  - Geo-related information in shapefiles (AHN, zandgeulenkaarten, geologische kaarten, etc.)

Explanation

The main functionality of D-Soil Model is to let the user combine different information about the subsoil, to make a schematization of the subsoil.

Not all information is digital or suitable to show on a screen (example: resp. analog cpt and geotechnical lengteprofiel).

Actions to support

Import data

View data on map

View data in table

View data in a drawing (cross section)

### Import data

Dataformats to import:

| File | Data | View |
| --- | --- | --- |
| *.sti | Soilprofile and soilparameters | Table I Map I Cross section |
| *.mdb | Soilparameters, see 'Schematiseringshandleiding' of relevant failure mechanism. | Table |
| soilsegments.csv | Soilprofiles with probability | Table I Map I Cross section |
| soilprofiles.csv | Layers, soilmaterial and yield stresses | Table I Cross section |
| surfacelines.csv | X,y,z coordinates | Table I Cross section |
| characteristicpoints.csv | X,y,z coordinates | Table I Cross section |
| *.gef | CPT, Boring | Map I Table/ Cross section |
| soilsegments.shp | Location of segments | Map |
| *.shp | Not defined | Map |

### All data can be imported separately, but two items are depending on others (cannot exist without the other):

Characteristic points depends on surfacelines.

Soilsegments depends on soilprofiles.

### View data on map

If data contains coordinates the information must be shown on a map with labels.

The map must have a background map (topografische kaart, like PDOK).

Other shapes then the WTI-SOS 2017 shape (segments.shp) can be shown on the map.

### View data in table

All imported data must be visible in a table, per row one item and information about this item in colums.

### View data in a drawing (cross section).

Soilprofiles

1D Soilprofiles of WTI-SOS 2017 must be visible in a drawing so the user can see the layers in different (adjustable) colors.

2D Soilprofiles (*.sti-files) must be visible in a drawing so the user can see the layers in different (adjustable) colors.

The layers and their minimum and maximum values must be clear in the drawing

Surfacelines

Surfacelines must be visible in a drawing.

## Use case 2 User must be able to combine subsoil information and view the combination

Explanation

The main functionality of D-Soil Model is to let the user combine different information about the subsoil, to make a schematization of the subsoil. Not all combinations are usefull.

In table below are the combinations mentioned of the base item and the item to add (blue).

|  | Cpt | Boring | 1D Soil profile | 2D Soil profile | Soil segment | Surfaceline |
| --- | --- | --- | --- | --- | --- | --- |
| Cpt |  |  |  |  |  |  |
| Boring |  |  |  |  |  |  |
| 1D Soil profile | x | x | x |  |  | x |
| 2D Soil profile | x | x | x |  |  | x |
| Soil segment (stability) |  |  |  |  |  | x |
| Surfaceline |  |  | x |  |  |  |

Actions to support

Connect items in the cross section (one to one)

Connect items in batch (several to one)

View combination in a drawing.

### Connect items in the cross section (one to one)

CPT and borings cannot combined starting with a cpt or boring.

1D Soil profile can be combined with:

- Cpt

- Boring

- 1D Soil profile

A 1D Soil profile needs to be combined with other 1D Soil profiles of the same soil segment.

- Surfaceline

Combining a 1D Soil profile with a surfaceline results in a 2D Soil profile. So the combination creates a new item.

2D Soil profile can be combined with:

- Cpt

- Boring

- 1D Soil profile

- Surfaceline

The combination of a 2D Soil profile and a surfaceline results in a new 2D Soil profile.

Soil segments (stability) can be combined with:

- Surfaceline

The combination of a segment and a surfaceline results in a segment with all new 2D Soil profiles, change of appearance stays unchanged.

A surfaceline can be comined with:

- 1D Soil profile

Combining a surfaceline with a 1D Soil profile results in a 2D Soil profile. So the combination creates a new item.

### Connect items in batch (several to one)

The combination of cpt/borings to a segment can be done in batch, not only one by one.

The user can make a selection of the cpts or borings in the table or on the map and connect them to a segment.

### View combination in a drawing.

To all combinations it is important that the Z-values remain intact (in m +NAP); show items at the right height. There is no definition of the x-value of 1D profiles, so in horizontal orientation is at the same leftside origin.

For some combination there are more requirements:

1D profile – other 1D profiles of the soil segment

All 1D profiles of one soil segment must be visible at the same time. With large (>6) numbers of 1D profiles a ‘filmstrip’ is provided, so the user can scroll through the 1D profiles.

2D profile -  cpt and boring

The cpt and boring are placed in the middle of the 2D soil profile. The user can drag them to the desired X-coordinate.

Note: This can be automated when the referentielijn is equal to the segments.shp or a new referenceline is introduced. This is not implemented in 16.1.

2D profile -  surfaceline

When combining 2D profiles and surfacelines there is a mapping necessary for the x-coordinates.

This mapping is done based on the referentielijn. The user can define this X-value in the 2D profile. On the surfaceline is the referentielijn situated at the middle of the characteristic points Kruin buitentalud and Kruin binnentalud.

The behaviour when the length of 2D profile and surfaceline is not equal is described in appendix X.

Soil segment – Cpt/borings

When a soil segment is combined with cpt and borings a drawing of the soil segment is needed. The soil segment is drawn as a straight line and the cpts and borings are situated relatively among this line. This is called a lengteprofiel.

<insert drawing of cpt’s mapped on the segment>

## Use case 3 User must be able to adept the WTI-SOS 2017 to local situation.

Explanation

The WTI-SOS 2017 is a general schematization of the subsoil at the innertoe of the primary dikes in the Netherlands. To perform the assessment it is needed that this schematization is adepted to the local situation (more detailed).

Actions to support

Save a selection of the WTI-SOS 2017 as a new project

Splitting the soil segments per failure mechanism

Deleting soil profiles per segment

Changing chance of occurance from the soil profiles

Make 2D soilprofiles

Adding 1D and 2D soilprofiles to a soil segment

### Save a selection of the WTI-SOS 2017 as a new project

The user can make a selection of segments (per failure mechanism, or all) and save this as a project. All soilprofiles of these soil segments are included in this project, as well as all their materials. Also connected cpts and borings are included in this project.

### Splitting the soil segments per failure mechanism

The user can add a split location to a soil segment in the map or in the length profile. When splitting the soil segment, all soilprofiles and their probability of occurance are connected to both new soil segments. The name of the new segments consists of the old name and a number.

### Deleting soil profiles per soil segment

The user can remove soil profile(s) from a soil segment. If the total probability of occurance is not 100%, a message should be given.

### Changing chance of occurance from the soil profiles

The user can change the probability of occurance of soil profile(s) from a soil segment. If the total probability of occurance is not 100%, a message should be given.

### Make 2D soil profiles

The user can make 2D Soil profiles either from scratch, either from using 1D Soil profile.

### Adding 1D and 2D soil profiles

The user can add soil profile(s) to a soil segment. If the total probability of occurance is not 100%, a message should be given.

## Use case 4 Make subsoil schematization for the failure mechanism Macrostabiliteit binnenwaarts (STBI)

Explanation

For the assessment the user must be able to define the subsoilschematization for the failure mechanism Macrostabiliteit binnenwaarts (STBI).

Actions to support

Save soil segments for the failure mechanism Macrostabiliteit binnenwaarts (STBI).

Define aquifer layers per soil profile.

Define soil parameters for the failure mechanism Macrostabiliteit binnenwaarts (STBI).

Define yield stresses per 2D soil profile (optional for undrained calculations).

### Save soil segments for the failure mechanism Macrostabiliteit binnenwaarts (STBI).

The user must be able to save the soil segment with all their connected data, such as the soil profiles (including yield stresses), materials, cpt and borings.

### Define aquifer layers per soil profile.

The user must be able to define a layer as a aquifer. A soil profile can contain more than one aquifer.

### Define soil parameters for the failure mechanism Macrostabiliteit binnenwaarts (STBI).

The user must be able to define the soil parameters needed for the failure mechanism Macrostabiliteit binnenwaarts (STBI). These soil parameters are defined in appendix X.

### Define yield stresses per 2D soil profile (optional for undrained calculations).

The user can define yield stresses in a 2D soil profile.

## Use case 5 Make subsoil schematization for the failure mechanism Piping (STPH)

Explanation

For the assessment the user must be able to define the subsoil schematization for the failure mechanism Piping (STPH).

Actions to support

Save soil segments for the failure mechanism Piping (STPH).

Define 1D soil profiles per soil segment.

Define aquifer layers per soil profile.

Define soil parameters for the failure mechanism Piping (STPH).

### Save soil segments for the failure mechanism Piping (STPH).

The user must be able to save the soil segment with all their connected data, such as the soil profiles, materials, cpt and borings.

### Define 1D soil profiles per soil segment.

When 2D soil profiles are used, the user must be able to define a failure mechanism location where a 1D soil profile is available for the piping kernel.

### Define aquifer layers per soil profile.

The user must be able to define a layer as a aquifer. A soil profile can contain more than one aquifer.

### Define soil parameters for the failure mechanism Piping (STPH).

The user must be able to define the soil parameters needed for the failure mechanism Macrostabiliteit binnenwaarts (STBI). These soil parameters are defined in appendix X.

UC Save and export data from D-Soil Model

## Use case 6 Use schematization in other applications

Explanation

D-Soil Model is developed for the WTI-2017. Therefore it is necessary that the schematizations from D-Soil Model can be used in Ringtoets and the Stand alone tools (Basis Module Macrostabiliteit).

Actions to support

Make a selection of the soilparameters needed for the use in:

- Ringtoets for failure mechanism Piping (STPH).

- Ringtoets for failure mechanism Macrostabiliteit binnenwaarts (STBI).

- BM - Macrostabiliteit for failure mechanism Macrostabiliteit binnenwaarts (STBI).

- BM - Piping for failure mechanism Piping (STPH).

### Make a selection of the soilparameters.

The usage of a D-Soil Model project in Ringtoets or the Basis Modules differs in the use of deterministic or stochastic values of soil parameters. The user must be able to see this difference so he/she knows which parameter are used.

In appendix B an overview is given of the needed parameters per usage.

## Error handling

In case of malfunction of the program the user must be informed via messages.

When this malfunctioning is due to data errors, the user must be informed in such a way that he knows how to adjust which data.

# References

- 1209429-001-GEO-0011-r-Uitgangspunten WTI 2017, H. Knoeff, Hans de Waal, okt 2014

N:\Projects\1209000\1209429\C. Report - advise\3- uitgangspuntennotities \1209429-001-GEO-0011-r-Uitgangspunten WTI 2017.docx

- Bokma, J et al. (2015). WTI D-Soil Model- Technical Design, Deltares report 1209430- 003-DSC-0021. (will be updated).

https://repos.deltares.nl/repos/Ringtoets/trunk/doc/WTI/DSoilModel/System/DSoilModel - Technical design.docx

3	Trompille, V (2014). WTI D-Soil Model- Testplan, Deltares report 1209430-003-DSC-0008.

https://repos.deltares.nl/repos/Ringtoets/trunk/doc/WTI/DSoilModel/System/DSoilModel - Test plan.docx

4 	Technical design Macro Stability kernel.

https://repos.deltares.nl/repos/FailureMechanisms/FailureMechanisms/DikesMacroStability/trunk/doc/DikesMacroStability Kernel - Requirements and Functional Design.pdf

5 	Technical design Piping kernel. In preparation

https://repos.deltares.nl/repos/FailureMechanisms/FailureMechanisms/DikesPiping/trunk/doc/DikesPiping Kernel - Requirements and functional design.pdf

6	Terminologielijst WTI2017

P:\1200109-sbw-algemeen\J. Uitvoeringsjaar 2015\16-Parameterlijsten en basisinformatie\Terminologielijst

###### Glossary

Following glossary is a selection from Terminologielijst [6]

| Term | Definition |
| --- | --- |
| Aquifer | Soil layer with a high permeabality compared to aquitards (i.e. Sand layers). |
| Aquitard | Soil layer with a low permeabality compared to aquifers (i.e. clay layers). The horizontal flow in a aquitard is very low, while a vertical flow could be significant. |
| Failure mechanism | The way a contruction fails due to high loads or load effects which are larger than the strength and what reduces the water retaining capacity. |
| Inner crest line | Line marking the transition between crest and slope. |
| Inner slope | Sloping face of the embankment at the innerside. |
| Inside toe | Lower boundary of the embankment at the land side (transition to ground level) |
| Outside slope | Sloping face of the embankment at the water side. |
| Outside toe | Lower boundary of the embankment at the water side (transition to ground level or foreland) |
| Deterministic | During assessment the parameter is considerd as 'known' and not modelled as a stochast. |
| Dike | A dike as part of a defence system. |
| Failure mechanism | Proces that leads to the failure of the system. This is often the result of a sequence of partial processes, derived from partial failure or the system, but without complete function loss. |
| Geometry | Collection of points in RD coordinates on a straight line, perpendicular on the dikeline with a heigth in m+NAP, which represent the geometry of the dike. Outside/riverside is always visualized on the left side. |
| Characteristic points | Characteristic points of the surface level, right-angled at the dike. |
| Crest | 1. Area between inner and outer crest. 2. Highest point in dike geometry 3 Outercrest line |
| Crest inner slope | The intersection point of the slope at the polder side and the crest of the dike |
| Macro stability | The mechanism that causes slope shear failure of dike embankments |
| Subsoil profile | A sequence of subsoil layers, either 1D or 2D. |
| Subsoil scenario | subsoil scenario with certain occurrence probability, in case of stochastic subsoil modelling. |
| sub soil schematization | A schematic reproduction of (a part of) the subsoil for a certain area. |
| Probabilistic analysis | Probabilistic determination of failure probability, using stochastically distributed variables for loading and strength. |
| stochastic (soil) scenario | The soilprofile, together with its chance of occurace, is a scenario. |
| (soil) segment | In the SOSWTI2017 the dikes are divided in segments. Soil segments are line elements (soil areas are polygons).To each segment soilscenario's with possible soilprofiles are assigned. |
| Stochastic variable | Stochastic variable is a variable whose value is subject to variations due to chance. |
| Stochastic (as in SOSWTI2017) | 'Stochastic' means that a certain aspect is variable. These variability is visible in the chances of occurance of the soilprofiles. |
| Toe dike inner side | The intersection point of the surface level at the dike side and the slope of the shoulder at the inner side (if no berm: dike) |
| Toe dike outer side | The intersection point of the slope of the berm at the outer side and the surface level at the outer side |

###### Soil material parameters for kernels

Filter soil material parameters for Macrostabiliteit – Basis Module [4] in D-Soil Model:

| WTI-ID (STBI-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. | Undetermined | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. |
| M04 | unsat | [kN/m3] | Unit weight of soil above phreatic level | Onverzadigd gewicht | D | 0.001 | 0.001 | ∞ |
| M03 | sat | [kN/m3] | Unit weight of soil below phreatic level | Verzadigd gewicht | D | 0 | 0 | ∞ |
| M05 | c' | [kN/m2] | Cohesion | Cohesie | D | NaN | 0 | ∞ |
| M06 | ’ | [˚] | Friction angle of shearing resistance | Hoek van inwendige wrijving | D | NaN | 0 | 89 |
| xx | su | [kN/m2] | Undrained shear strength | Ongedraineerde schuifsterkte | D | NaN | 0 | ∞ |
| M14 | S | [-] | Undrained shear strength ratio (normally consolidated) | Verhouding Su/Pc | D | 0.22 | 0 | ∞ |
| M15 | m | [-] | Strength increase exponent | Sterkte toename exp (m) | D | 0.8 | 0 | 1 |
| xx | POP | [kN/m2] | Pre overburden pressure | POP | D | NaN | 0 | ∞ |
| xx | Shear strength model | n.a. | Shear strength model (choice between C-Phi, Su-calculated and Su-measured) | Schuifsterkte model | n.a. | C-Phi | n.a. | n.a. |
| n.a. | Use POP | n.a. | Only relevant if the “Su-calculed” shear strength model is selected: TRUE = "Su-calculated with" and FALSE "Su-calculated with yield stress". | Gebruik POP | n.a. | False | n.a. | n.a. |

Filter soil material parameters for Piping - Ringtoets in D-Soil Model:

| WTI-ID (STPH-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. | n.a. | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. | n.a. | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. | n.a. | n.a. | n.a. |
| M13 | D70 | [m] | 70% fractile of the aquifers grain size distribution | Diameter D70 | S –lognormaal | n.a. | 1E-8 | 1 |
| M15 | DarcyPermeability | [m/s] | Permeability (Darcy) | Doorlatendheid | S | n.a. | 0 | 1000 |
| M07 | PipingLayer.AbovePhreaticLevel | [kN/m3] | Unit weight above phreatic level | Onverzadigd gewicht | S - lognormaal | 10 | 0.001 | 100 |
| M08 | PipingLayer.BelowPhreaticLevel | [kN/m3] | Unit weight below phreatic level | Verzadigd gewicht | S – verschoven lognormaal | 0.01 | 0.001 | 100 |

Soil segment is an area or a line with consistent subsoil schematization

Soil profile is a cross section of the subsoil described in layers of material

To plot a *.sti on a map, coordinates of one point of the surfaceline is needed, as well as the location of this point on the surfaceline, i.e. only coordinates is not sufficient

The application can import borehole and CPT as GEF-file. The location of the borehole and CPT can be viewed in

map. If the location in RO is known, it is plotted in the right location. If the RO location is not known, it is not plotted on the map or at (60000, 500000).