# Section 1: Data extraction

Task 1a: Extract wave parameters (wave height, peak period, wave direction) at locations in 20m depth along the Dutch coast

- lon lat in: \trunk\doc\SDToolBox – Test plan\study_case_NL\data\NS_WAM_ERA5.nc

Verification: plot temporal series

Task 1b: Extract wave parameters for year 1986 in a gridded box (longitude: 250 to 260 lat: -5 to 5, intervals each 2 degrees).

Verification: Plot with spatial maps of the mean values and standard deviation

Task 2: Extract mean sea level pressure from ERA5 dataset on a gridded box on the North Sea.

lonmin=-5;lonmax=11;dlon=0.5

latmin=50;latmax=62;dlat=0.5

Verification: for a point in that grid plot temporal series. Spatial map of the mean sea level pressure.

Task 3: Test functionality to generate output netcdf files for waves and sealevel pressure

# Section 2: Data preprocessing

Task 4: Compute the squared spatial gradients of mean sea level pressure (this function is under predictor_definition library, compute_spatial_gradients)

Verification: for a point in that grid plot temporal series. Spatial map of the spatial gradient of mean sea level pressure.

Task 5: Temporal resampling

Wave climate data: hourly to 3hourly

Mean Sea Level pressure and gradient: hourly to 6hourly

Verification: for a point in that grid plot temporal series. Spatial map of the mean sea level pressure and gradient.

Task 6: Spatial resampling

Mean Sea Level pressure: 0.5 degree to 1 degree

Verification: for a point in that grid plot temporal series. Spatial map of the mean sea level pressure and gradient.

# Section 3: Predictor definition

Task 7a: Aggregation of atmospheric data correlated with wave generation, use averaged during 1 day. For computing daily running means of the 6h spatial fields. (function atmospheric_averaged_mean in predictor_definition library)

Task 7b: If we are good with time, repeat task 7a with aggregation data option.

Verification: for a point in that grid plot temporal series. Spatial map of the mean sea level pressure and gradient.

Task 8a: Perform Principal Component Analysis on the temporal and spatial fields of mean sea level pressure and the squared gradient. The function returns the Empirical orthogonal functions (EOF) and the principal components (PCs) the function is in the predictor definition library (compute_PCA).

Verification: plot EOFs (spatial map) and PCs (temporal series)

# Section 4: Statistical model

Task 8b: Compute Principal Component Analysis on the different coastal locations with wave climate (if we are doing well with time). If this task is not done, continue selecting only one wave station in the coast.

Verification: plot EOFs (spatial map) and PCs (temporal series)

Task 9: Normalization of the predictor data (X, they will be the PCs) using Z score method (X= (PCs-mean)/std, there is a function for this)

Task 10: Define calibration and validation subsets with stratified K folds, try with 5 splits (80% calibration; 20% validation) and 2 repetitions to start with. (Find more details that I have written in https://issuetracker.deltares.nl/browse/SDTOOLBOX-41)

Verification: Plot temporal series of wave climate selected for calibration and validation (can be using different colors on the same plot).

Task 11: Train the multivariate regression model (Y=alpha0+sum(alphai*Xi)), the predictand Y = Significant wave height, the predictor X= normalized PCs

Verification: Check which PCs are significative (which numbers are in the model), Plot scatter X,Y empirical and X,Y* modelled, for the training subset and the validation subset (you can do this very simply using the average of the coefficients of the 8 models defined, (5-1)splits*2repetitions)

Task 12: Compute accuracy scores for the stratified folds taken for validating (there are functions defined for this in the library statistical model, get_all_scorers())

Verification: table with scores

Task 13: Repeat 10, 11 and 12 but with wave direction, because the regression model for circular variables is different

# Section 4: Wave future projections

Task 14: Retrieve EC-Earth atmospheric data and preprocess spatially and temporally as in ERA5 (task 4,5,6,7)

Task 15: Correct systematic errors in EC-Earth data compared to ERA5 (bias correction in preprocessing library)

Task 16: Project atmospheric data of EC-Earth in the EOF defined in task 8a

Task 17: Normalize the Principal components of the EC-Earth with the mean and std of the ERA5 (obtained in task 9)

Task 18: Project wave climate with the EC-Earth atmospheric data using the trained multivariate regression model in Task 11.