# Bijlage Functioneel ontwerp Taludlaag Tool

Voor alle technische documentatie met betrekking tot software wordt bij Deltares de engelse taal gehanteerd. Vandaar dat deze bijlage in het engels is.

## Functional design Taludlaag Tool (DAMSlopeLayerTool)

Goal: Combination of surface line with a 2D soilprofile with a slope layer

The user can define an extra soil layer between the characteristic points Dike crest at river and Dike toe at polder with:

- thickness of layer [m], see slope_layer_thickness in OverviewDataUIAndEngine.xlsx

- material of layer; see slope_layer_material in OverviewDataUIAndEngine.xlsx

The bottom level of the extra layer is a copy of the surfaceline with an offset between points Dike crest at polder and Dike toe at polder. The offset is defined by the thickness of the layer.

The layer will be ‘filled’ with given material; parameter slope_layer_material

Both new parameters can be defined per location.

Figure 1 Profile with characteristic points

Figure 2 Profile with slope layer, defined by slope_layer_thickness (red arrow)

## User Interface

No UI, but a command line tool

Usage:

AddLayerOnSlope /B /I <Input folder> /O <output folder>

Usage:

- <input folder> is folder for import files

- <output folder> is folder generated files

The import files will be read from the specified input location and the generated files (*.sti) will be written to the specified output location. The results will always be a set of 2d geometries (*.sti).

## Import Files

These are the files needed for the import.

1. Location.csv (for location properties) (different from normal DAM import)

- locationId: string

- surfaceline_id: string

- soilgeometry2D_name: string (for2D), output of ‘Onder-naast Tool’/DamSoilProfileTool

- x_soilgeometry2D_origin

- slope_layer_thickness: float

- slope_layer_material: string

2. characteristicpoints.csv (same as normal DAM import)

3. surfacelines.csv (same as normal DAM import)

4 Use Map Geometries containing *.sti files (D-Geo Stability files)

New parameters

Two new parameters are introduced:

|  | Thickness slope layer | Material slope layer |
| --- | --- | --- |
| Name (in defx) | slope_layer_thickness | slope_layer_material |
| Column csv / attribute shape | slope_layer_thickness | slope_layer_material |
| Type | Float | StringId |
| Unit | [m] | [-] |
| Default | 0 | none |
| Min value | 0 | none |
| Max value | 5 | none |