Trefwoorden

Stochastic subsoil schematization, soil profile, soil properties, macro stability, piping, flow slide

Samenvatting

This document contains the requirements and a global description of the functional design for the application D-Soil Model, which will be a part of the WTI 2017 software. The application supports the (stochastic) subsoil schematizations for the failure mechanism macro stability, piping and flow slide. A subsoil schematization for an area or segment consists of 1D/20 soil profiles and soil material properties.

| Versie | Datum | Auteur | Paraaf | Review | Paraaf | Goedkeuring | Paraaf |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | maa. 2016 | Irene van der Zwan |  | Kin Sun Lam |  | Maya Sule |  |

Status concept Dit document is een concept en uitsluitend bedoeld voor discussiedoeleinden. Aan de inhoud van dit rapport kunnen noch door de opdrachtgever, noch door derden rechten worden ontleend.

Inhoud

1	Introduction	1

1.1	Purpose and scope	1

1.2	Other system documents	1

2	Capabilities of the application	2

2.1	General constraints	2

2.2	Capabilities related to the failure mechanisms	2

2.3	Capabilities related to the interface	2

2.4	Items that are not supported	2

3	Requirements	3

3.1	General and non-functional requirements	3

3.2	Functional requirements	3

3.2.1	Following out of CNS 3	3

3.2.2	Following out of CNS 4	3

3.2.3	Following out of CNS 5	3

3.2.4	Following out of CNS 6	3

3.2.5	Following out of CNS 7	4

3.2.6	Following out of CNS 8	4

3.2.7	Following out of CNS 9	5

4	Functional design	6

4.1	Functional description	6

4.2	Interfaces	6

4.3	Use cases	6

4.4	Error handling	12

5	References	13

Bijlage(n)

A	A-1

# Introduction

## Purpose and scope

This document contains the requirements and the functional design for the application D-Soil Model, which will form a part of the WTI 2017 software applications. The application supports at least the (stochastic) subsoil schematizations for the failure mechanism macro stability and. A subsoil schematization for an area or segment consists of soil profiles and soil material properties.

D-Soil Model is a software tool for schematization of the subsoil and is developed for the assessment of primary dikes according to the WTI2017. In this process of assessment the schematization of a stochastic subsoil model (SOSWTI2017) is a major source of information. And the D-Soil Model development focusses on use of this stochastic subsoil model (SOSWTI2017).

The document will not give any background on the context of the WTI project and on the derivation or motivation of the supported schematizations. For this purpose the reader is referred to the VTV 2017 and to its supporting technical reports and their background reports underneath.

This document is a revision of 1209430-003 WTI 2017 D-Soil Model Requirements and Functional Design and is applicable to version 15.1. In chapter 2 the constraints are described. These are translated to requirements, chapter 3. Chapter 4 describes the functional design of D-Soil Model (actual version and not implemented functionality). The functional design describes the functionality via user cases; the answer to ‘what?’. The answer to ‘How?’ is given in the Technical Design.

A glossary of used terms is placed in appendix A.

## Other system documents

For the development of D-Soil Model different documents are available. The titles are posted in the table below.

| Title | Content |
| --- | --- |
| Requirements and functional design | This document |
| Technical Design [2] | Definition of the different software components and their interaction |
| Test plan and Test report [3] | Description of the different regression and acceptation tests, including target values. |

# Constraints

## General constraints

CNS 1. 	As a general constraint, the development process needs to comply with the the Deltares DQMS system.

CNS 2. 	As a general constraint, the application needs to comply with the relevant general requirements and further design rules for the programming, documentation and testing of WTI software. This set of requirements and rules are available in a separate document [1].

## Capabilities related to the failure mechanisms

CNS 3. 	Import data from existing data formats.

CNS 4.	D-Soil Model must provide all soil parameters needed for the stability- and piping calculations in the WTI software.

CNS 5.	The application is at least suitable to support the failure mechanisms Piping, Macro Stability and Flow Slide.

CNS 6.	The schematization of the subsoil for these mechanisms must be complete and the calculation software can use the subsoil schematizations directly.

CNS 7.	The application is applicable for stochastic subsoil modeling.

## Capabilities related to the interface

CNS 8.	The user must have control to view the building blocks of the subsoil schematization.

CNS 9.	The user must have control to edit the building blocks of the subsoil schematization.

## Items that are not supported

CNS 10.	No data related to water pressures are stored in the application. Only parameters that are directly related to the soil.

CNS 11.	Surface lines and characteristic points are only for reference and not suitable for calculation purposes in calculation software, they are only used for internal purposes.

# Requirements

This chapter describes the requirements that follow out of the constraints. Most requirements are too general to be programmed directly. Chapter 4 specifies the requirements further as a basis for the following technical design in user stories.

## General and non-functional requirements

The externally defined general and non-functional requirements for all WTI software apply;

see section 2.1 where CNS 1 and CNS 2 are defined.

## Functional requirements

### Following out of CNS 3

Data to import consists of data needed for WTI software and data to support the subsoil schematization by the user (such as polder water level, surface lines and characteristic points).

REQ 3 Format of import data.

The required import files are:

- sti file and mdb file from D-GeoStability; for subsoil schematizations

- (2D profiles) and strength parameters.

- csv files; description of the segments and 1Dsoil profiles.

- shape files; location of segments, 2D profiles, CPT's, borings.

### Following out of CNS 4

REQ 4 Soilparameters

All soil parameters needed for the stability- and piping calculations in the WTI software must be incorporated in D-Soil Model.

### Following out of CNS 5

REQ 5 The  application  gathers  the  (stochastic)  subsoil  schematizations  input  and parameters for the following kernels:

- WTI macro stability kernel.

- WTI piping kernel.

- WTI flow slide kernel.

D-Soil Model can save this output into one file.

### Following out of CNS 6

REQ 6 Schematization of subsoil

For each failure mechanism the subsoil is described in soil segments consisting of one of more scenario(s) of a soil profile with a probability of appearance. These soil profiles are described in either 1D or 20 layer schematization. A soil segment can have a combination of

1D and 20 soil profiles. Each layer is described with at least all the parameters needed for the relevant failure mechanism. This output is gathered in one file, which can be directly imported in WTI software.

### Following out of CNS 7

REQ 7 Stochastic schematization of subsoil.

All the parameters of above described schematization can consist as a stochastic variable or "as is" value (characteristic value or design value) or as both. D-Soil Model stores both values. The program using D-Soil Model projects can decide which one to use for which calculation; the two values are not connected to each other; changing one does not change the value of the other.

The stochastic variables can be stored as a mean value with a distribution type and the distribution itself. An overview of al variables is given in appendix X.

### Following out of CNS 8

The building blocks of the subsoil schematization are:

SOSWTI2017

- Soilsegments

- Location on map

- Failure mechanism

- Scenario of soil profiles with the probability of appearance

- Soilprofiles

- Profile data: yield stresses

- Layer data: material name, boundaries, aquifer/aquitard

- Soilmaterials

- Soilparameters per material name

Supporting data (optional)

- CPT

- Boring

- Lab data Hydraulic levels

REQ 8 View the building blocks of the subsoil schematization

The subsoil schematization must be visible to the user:

All data is shown in tables to provide an overview and edit possibilities.

Data with a geographical reference (segment, CPT, boring, etc.) is shown on a map (with legend and label-option).

Schematization of layers or other graphs (e.g. CPT) is visible as cross sections.

Link to file-related programs

Depending on the kind of file format, the files can also be viewed in a format-related program;

e.g. *.sti (2D profiles) in DGeoStability, *.gef (CPT and borings) in a GEF viewer.

### Following out of CNS 9

REQ 9 Edit and generate the building blocks of the subsoil schematization

The user is able to edit all the building blocks. The user is responsible for the edits, D-Soil Model gives validation messages when applicable (e.g. minimum and maximum values).

# Functional design

## Functional description

D-Soil Model is a software tool for schematization of the subsoil and is developed for the assessment of primary dikes according to the WTI2017. In this process of assessment the schematization of a stochastic subsoil model (SOSWTI2017) is a major source of information. And the D-Soil Model development focusses on use of this stochastic subsoil model (SOSWTI2017). The user must be able to make a local schematization of the SOSWTI2017 schematization. Therefor the user must be able to :

- combine the SOSWTI2017 with subsoil investigation (CPT and borings)

- split SOSWTI2017 segment

- change subsoil scenario’s (profiles and their chance of presence)

- make 2D profiles

- manage material parameters

## Interfaces

D-Soil Model must provide subsoil information to the WTI software. The schematization, made in D-Soil Model, can be imported in the WTI software.

Each WTI application can choose which data to use from the D-Soil Model file.

## User story

The user stories are linked to the requirements in chapter 3. Each user story is related to at least one requirement and one requirements can link to several user stories (minimal one).

All the user stories are formulated as ; As a …. I want…. so that I can…..

Most (all) user stories concern the roll of subsoil modeller.

US 3.1 Import subsoil data

As a modeller I want to import the following data in these format, so that I can combine data to make a subsoil schematization.

| File | Data | Shown |
| --- | --- | --- |
| *.sti | Soilprofile and soilparameters | Table I Map I Cross section |
| *.mdb | Soilparameters, see 'Schematiseringshandleiding' of relevant failure mechanism. | Table |
| soilsegments.csv | Soilprofiles with probability | Table I Map I Cross section |
| soilprofiles.csv | Layers, soilmaterial and yield stresses | Table I Cross section |
| surfacelines.csv | X,y,z coordinates | Table I Cross section |
| characteristicpoints.csv | X,y,z coordinates | Table I Cross section |
| *.gef | CPT, Boring | Map I Cross section |
| soilsegments.shp | Location of segments | Map |
| *.shp | Not defined | Map |

Figure A. 1  Import files D-Soil Model

US 3.2 Import and view all SOSWTI2017 data

As a modeller I want to be able to get an overview of all available SOSWTI2017 subsoil schematizations  by browsing through them so that I get a good visual overview of all available soil profiles and their probabilities.

US 3.3 Select relevant SOSWTI2017 information for project

As a modeller I want to use only the relevant part of the SOSWTI2017 by defining the relevant area, a reference line so that I am not distracted by information I do not use. In other words I only want to use (see and save) my area.

US 4.1  Soilparameters

As a modeller I want to see the soilparameters needed for the stability and piping calculations in WTI-software. The list of these soilparameters is given in appendix A.

US 5.1 Supply the subsoil schematization and parameters for the WTI macro stability kernel.

As a modeller I want to schematize the subsoil for the failure mechanism macro stability so that I can use the schematization for calculations in the macrostability kernel.

All inputs of the macrostability kernel are defined in the technical design document of the kernel. [4].

US 5.2 Supply the subsoil schematization and parameters for the WTI piping kernel.

As a modeller I want to schematize the subsoil for the failure mechanism piping so that I can use the schematization for calculations in the piping kernel.

All inputs of the piping kernel are defined in the technical design document of the kernel. [5].

US 5.3 Supply the subsoil schematization and parameters for the WTI D-Flow slide kernel.

As a modeller I want to schematize the subsoil for the failure mechanism flow slide so that I can use the schematization for calculations in the flow slide kernel.

US 5.4 Supply all relevant subsoil

As a modeller I want to have an overview of the general parameters [8] and the parameters per failure mechanism and application (see appendix B) that have to do with subsoil information so that I can change and store the parameters. Filters are not implemented in 15.1)

US 6.1 Stochastic characterization of the variables

As a modeller I want to add stochastic variables with a distribution to a material so that I can perform probabilistic calculations in the kernels macrostability, piping and flow slide.

US 6.2 Determination of soil parameters (will not implemented in version 16.1)

As a modeller I want to automatically derive the stochastic parameters from a set testing data (e.g. field or lab data) for a given distribution so that I don’t have to this by hand. For example the mean and standard deviation for a lognormal distribution for a given set lab data.

US 6.3 Separate segments for each failure mechanism

As a modeller I want to change the SOSWTI2017 segment boundaries per  failure mechanism so that I can make a schematization per failure mechanism. Each mechanism has its own segment boundaries.

US  6.4 Definition of the location of the 1D profile for Piping and DFlowSlide

As a modeller I want to point out the location of calculation for 1D failure mechanisms (piping and flowslide) in an existing 2D profiles (macrostability) so that I don’t have to make a new profile for 1D calculations.

US 6.5 Profile properties

As a modeller I want to add, change and store the soil property yield stress (Pre-consolidation stress) so that I can use the schematization for calculations with undrained soil (stability kernel).

US 6.6 Layer properties

As a modeller I want to add, change and store the layer property ‘Is aquifer’ so that I can use the schematization for calculations that use the waternet creator (stability and piping kernel).

US 7.1 Stochastic subsoil modeling

As a modeller I want to add, change and store the probability of appearance of the soilprofiles so that I can refine the stochastic soil schematization from SOSWTI2017.

US 7.2 Allocation of CPT's and boreholes on a map and to a segment

As a modeller I want to see the imported CPT's and borings on a map and match them to a segment so I can combine the information.

US 7.3 Overview of all available borings and CPT's in a segment or cross section

As a modeller I want to see all available ground investigation so I can pick a cpt or boring and combine it with a soil profile.

US 7.4 Overview of all available SOSWTI2017 profiles in a segment

As a modeller I want to see all SOSWTI2017 1D profiles per segment per mechanism so that can pick a cpt or boring and combine it with a soil profile.

US 8.1 Reference line

As a modeller I want to see the projection of soil investigation in longitudinal direction of a segment so I can see where the segment should be splitted.

US 8.2 Table view for soil parameters

As a modeller I want to see all the information in a tabular format so that I have an overview of the needed soilparameters and to change values easily.

US 8.3 Map view for soil areas (for areas will not implemented in version 16.1)

As a modeller I want to see in map with a detailed topographic map in the background the division of the soil segments or areas so I know where they are situated geographically and to check if there is no overlapping.

US 8.4 Profile view to modify soil profiles

As a modeller I want to see soilprofiles as a drawing so that I can see easily the thickness and material of layers.

As a modeller I want to see surfacelines as a drawing so that I can see easily the shape and the position of the characteristic points.

US 8.5 Automatically project boring and CPT onto a cross section (will not implemented)

This is US is mentioned cause it was forseen in earlier version but is invalid: cross sections do not have a geo reference. CPS or borings can there for not automatically projected on cross sections.

US 8.6 Show segments with labels

As a modeller I want to see the labels of the segments on the map so that I can see in one view which segment is where situated.

US 8.7 Show all soil investigation with labels in map view

As a modeller I want to see the labels of the boring or CTP on the map so that I can see in one view which boring or cpt is where situated.

US 8.8 Show surface lines on top of a 1D profile (characteristic points not implemented in version 15.1)

As a modeller I want to view surface lines and characteristic points on top of a 1D soil profile.

US 8.9 Select borings and CPT's onto a cross section

As a modeller I want to drag a borehole or CPT to a chosen location in the 2D profile so I can check the profile with the ground investigation. NB: The height reference of a borehole or CPT is assumed to be the same as in the soil profile and plotted in the height reference of the 2D profile.

US 8.10 1D Scenario overview

As a modeller I want to get an overview of the profiles and their probabilities of one segment so I can see <onderlinge verband> of the soilprofiles and the <verhoudingen> of the probabilities.

US 8.11 View files in file-related software. (will not implemented in version 16.1)

As a modeller I want to open while in D-Soil Model imported files in an external program so that I don’t have to go outside D-Soil Model for opening other programs.

This means file in GEF format (CPT's and borings) open in GefPiotTool and *.sti files (2D geometries) open in DGeoStability by doubleclicking the name in D-Soil Model.

US 8.12 Show data from AHN or a WMS server (WMS will not implemented in version 16.1)

As a modeller I want to see relevant background information as AHN information and information from a WMS server on the map so I can combine these information with the D-Soil Model data.

US 8.13 Show SOSWTI2017 1D profiles in 2D profiles

As a modeller I want to see combine relevant SOSWTI2017 1D profiles with a 2D profile at the e reference level so that I make sure the 2D subsoil schematization fits the SOSWTI2017 segments.

REQ 8.14 Draw SOSWTI2017 profiles of one segment on the same height for comparison

As a modeller I want the SOSWTI2017 profiles plotted next to one another at the same reference level (e.g. NAP) and scale so I can make a simple comparison. The minimum and maximum value of the layers must also be shown.

REQ 8.15 Show ground investigation simultaneously to REQ 8.14 at the same reference level

As a modeller I want to see the soil investigations (CPT's and borings) at the same reference level (e.g. NAP) so I can make a simple comparison.

REQ 8.16 Synchronization between different views

As a modeller I want to switch between different kind of views of the data by clicking on the name so I get an overview the information.

REQ 8.17    Visibility of yield stress

As a modeller I want to plot yield stresses in the 2D profile as points with optional labels (label =Spanningswaarde) so I can add yield stresses to the schematization.

REQ 9.1a Name soil materials

As a modeller I want to name or rename the materials and in a table view so I can easily manage this information.

REQ 9.1b Update soil materials

As a modeller I expect that the modification of the soil materials automatically updates the soil materials in the regarding soil layers so I don’t have to adept it myself.

REQ 9.1c Update soil materials

As a modeller I want to filter the material parameters on failure mechanism so I know which parameters to fill in.

REQ 9.2 Edit the chances of a SOSWTI2017 scenario

As a modeller I want to change the probably of appearance of one SOSWTI2017 scenario so I can make a local schematization of the SOSWTI2017.

REQ 9.3 Turn a 1D profile into a 2D profile

As a modeller I want to change an existing 1D profile into a 2D profile so that I can make changes in the cross section.

REQ 9.4 Draw 2D sti files

As a modeller I want to make changes in a cross section by adding new layers by drawing new boundaries (lines) in a 2D profile so I can make 2D schematizations in D-Soil Model for macrostability calculations.

REQ 9.5 Draw surface lines on 2D profiles (not implemented in 15.1)

As a modeller I want to plot surface lines and characteristic points plotted onto a 2D profile so I can make 2D schematizations in D-Soil Model for macrostability calculations.

REQ 9.6 Interpretation tools (will not implemented in 16.1)

As a modeller I want to interpretate CPT's directly into a 1D profile with strength parameters per material so I don’t have to do this in a separate tool.

REQ 9.7 Define soil layers

As a modeller I want to be able to schematize two dimensional soil layering so I can make 2D schematizations in D-Soil Model for macrostability calculations.

REQ 9.8 Allocate soil material to layer

As a modeller I want to allocate a soil material to a soil layer in one or more soil profiles so I schematize soil layers.

REQ 9.9 Define layer as aquifer or aquitard

As a modeller I want to define a soil layer in a soil profile as an aquifer so I can use the schematization in kernels which use the waternetcreator kernel.

REQ 9.10 Enter yield stresses for macro stability

As a modeller I want to enter pre-consolidation pressures in a 2D profile so I can make 2D schematizations in D-Soil Model for macrostability calculations.

REQ 9.11 Copy/ paste segments for one failure mechanism to the next

As a modeller I want to copy a segment from one mechanism to another mechanism so I don’t have to make new import data.

REQ 9.12 Modify the location of the borehole or CPT in the 2D profile

As a modeller I want to drag a CPT or boring to a chosen location in the 2Dprofile view so I can check the 2D schematization.

REQ 9.13 Logging of all changes to the database (will not implemented in version 16.1)

As a modeller I want to enter all relevant changes made to the input data so I keep track of the changes.

REQ 9.14 Minimal validity check on the data

As a modeller I want to know if my schematization is valuable to the calculation kernel so I can make my calculations with data errors.

REQ 9.15 Extensive validity check on the data (will not implemented in version 16.1)

As a modeller I want to get a complete validation for a Ringtoets analyses per failuremechanism so I know that my schematization is correct for Ringtoets.

REQ 9.16 Define n and gamma per material parameter for determination of stochastic or design values (will not implemented in version 16.1)

As a modeller I want to define and store the nature of the soil tests regarding so stochastic values can be calculated.

REQ 9.17 Store lab data (will not implemented in version 16.1)

As a modeller I want to store all laboratorium data used to derive soil parameters so that I can add extra test and update the soil parameters.

REQ 9.18 Build a 2D profile based on multiple 1D profiles (will not implemented in version 16.1)

As a modeller I want to be able to build a 2D profile based on multiple 1D profiles so I don’t have to draw a 2D profile completely from scratch.

REQ 9.19    Draw a new subsoil configuration (not implemented in version 15.1)

As a modeller I want to be able to configure a new subsoil schematization from scratch so I can make new schematizations.

REQ 9.20 Incorporate settlements underneath an embankment

As a modeller I want to be able to define settlements underneath the embankment so I can easily adept the 2D profile.

REQ 9.21 Visualize minimum and maximum of layer boundaries from the SOSWTI2017 data

As a modeller I want to see the expected value together with the minimum and maximum value of the layer boundary in the WTISOS2017 so I can make a local schematization of the WTISOS2017.

REQ 9.22 Split segments

As a modeller I want to split segments on the map or in a longitudal cross section of the segment so I can make a local schematization of the WTISOS2017.

REQ 9.23 Import soil areas and segments

As a modeller I want to import soil areas (will not implemented in version 16.1) and segments as shape files so I can see the location of these areas/segments.

REQ 9.24 Table import/export

As a modeller I want to be able to import and export the data from all tables so I can see and adjust the data in other applications, such as windows excel.

## Error handling

In case of malfunction of the program the user must be informed via messages.

When this malfunctioning is due to data errors, the user must be informed in such a way that he knows how to adjust which data.

# References

- 1209429-001-GEO-0011-r-Uitgangspunten WTI 2017, H. Knoeff, Hans de Waal, okt 2014

N:\Projects\1209000\1209429\C. Report - advise\3- uitgangspuntennotities \1209429-001-GEO-0011-r-Uitgangspunten WTI 2017.docx

- Bokma, J et al. (2015). WTI D-Soil Model- Technical Design, Deltares report 1209430- 003-DSC-0021. (will be updated).

https://repos.deltares.nl/repos/Ringtoets/trunk/doc/WTI/DSoilModel/System/DSoilModel - Technical design.docx

3	Trompille, V (2014). WTI D-Soil Model- Testplan, Deltares report 1209430-003-DSC-0008.

https://repos.deltares.nl/repos/Ringtoets/trunk/doc/WTI/DSoilModel/System/DSoilModel - Test plan.docx

4 	Technical design Macro Stability kernel.

https://repos.deltares.nl/repos/FailureMechanisms/FailureMechanisms/DikesMacroStability/trunk/doc/DikesMacroStability Kernel - Requirements and Functional Design.pdf

5 	Technical design Piping kernel. In preparation

https://repos.deltares.nl/repos/FailureMechanisms/FailureMechanisms/DikesPiping/trunk/doc/DikesPiping Kernel - Requirements and functional design.pdf

6	Terminologielijst WTI2017

P:\1200109-sbw-algemeen\J. Uitvoeringsjaar 2015\16-Parameterlijsten en basisinformatie\Terminologielijst

###### Glossary

Following glossary is a selection from Terminologielijst [6]

| Term | Definition |
| --- | --- |
| Aquifer | Soil layer with a high permeabality compared to aquitards (i.e. Sand layers). |
| Aquitard | Soil layer with a low permeabality compared to aquifers (i.e. clay layers). The horizontal flow in a aquitard is very low, while a vertical flow could be significant. |
| Failure mechanism | The way a contruction fails due to high loads or load effects which are larger than the strength and what reduces the water retaining capacity. |
| Inner crest line | Line marking the transition between crest and slope. |
| Inner slope | Sloping face of the embankment at the innerside. |
| Inside toe | Lower boundary of the embankment at the land side (transition to ground level) |
| Outside slope | Sloping face of the embankment at the water side. |
| Outside toe | Lower boundary of the embankment at the water side (transition to ground level or foreland) |
| Deterministic | During assessment the parameter is considerd as 'known' and not modelled as a stochast. |
| Dike | A dike as part of a defence system. |
| Failure mechanism | Proces that leads to the failure of the system. This is often the result of a sequence of partial processes, derived from partial failure or the system, but without complete function loss. |
| Geometry | Collection of points in RD coordinates on a straight line, perpendicular on the dikeline with a heigth in m+NAP, which represent the geometry of the dike. Outside/riverside is always visualized on the left side. |
| Characteristic points | Characteristic points of the surface level, right-angled at the dike. |
| Crest | 1. Area between inner and outer crest. 2. Highest point in dike geometry 3 Outercrest line |
| Crest inner slope | The intersection point of the slope at the polder side and the crest of the dike |
| Macro stability | The mechanism that causes slope shear failure of dike embankments |
| Subsoil profile | A sequence of subsoil layers, either 1D or 2D. |
| Subsoil scenario | subsoil scenario with certain occurrence probability, in case of stochastic subsoil modelling. |
| sub soil schematization | A schematic reproduction of (a part of) the subsoil for a certain area. |
| Probabilistic analysis | Probabilistic determination of failure probability, using stochastically distributed variables for loading and strength. |
| stochastic (soil) scenario | The soilprofile, together with its chance of occurace, is a scenario. |
| (soil) segment | In the SOSWTI2017 the dikes are divided in segments. Soil segments are line elements (soil areas are polygons).To each segment soilscenario's with possible soilprofiles are assigned. |
| Stochastic variable | Stochastic variable is a variable whose value is subject to variations due to chance. |
| Stochastic (as in SOSWTI2017) | 'Stochastic' means that a certain aspect is variable. These variability is visible in the chances of occurance of the soilprofiles. |
| Toe dike inner side | The intersection point of the surface level at the dike side and the slope of the shoulder at the inner side (if no berm: dike) |
| Toe dike outer side | The intersection point of the slope of the berm at the outer side and the surface level at the outer side |

###### Soil material parameters for kernels

Filter soil material parameters for Macrostabiliteit – Basis Module [4] in D-Soil Model:

| WTI-ID (STBI-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. |
| M04 | unsat | [kN/m3] | Unit weight of soil above phreatic level | Onverzadigd gewicht | D | 0.001 | 0.001 | ∞ |
| M03 | sat | [kN/m3] | Unit weight of soil below phreatic level | Verzadigd gewicht | D | 10 | 10 | ∞ |
| M05 | c' | [kN/m2] | Cohesion | Cohesie | D | NaN | 0 | ∞ |
| M06 | ’ | [˚] | Friction angle of shearing resistance | Hoek van inwendige wrijving | D | NaN | 0 | 89 |
| xx | su | [kN/m2] | Undrained shear strength | Ongedraineerde schuifsterkte | D | NaN | 0 | ∞ |
| M14 | S | [-] | Undrained shear strength ratio (normally consolidated) | Verhouding Su/Pc | D | 0.22 | 0 | ∞ |
| M15 | m | [-] | Strength increase exponent | Sterkte toename exp (m) | D | 0.8 | 0 | 1 |
| xx | POP | [kN/m2] | Pre overburden pressure | POP | D | NaN | 0 | ∞ |
| xx | Shear strength model | n.a. | Shear strength model (choice between C-Phi, Su-calculated and Su-measured) | Schuifsterkte model | n.a. | C-Phi | n.a. | n.a. |
| n.a. | Use POP | n.a. | Only relevant if the “Su-calculed” shear strength model is selected: TRUE = "Su-calculated with" and FALSE "Su-calculated with yield stress". | Gebruik POP | n.a. | False | n.a. | n.a. |

Filter soil material parameters for Macrostabiliteit - Ringtoets in D-Soil Model:

| WTI-ID (STBI-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. |
| M04 | unsat | [kN/m3] | Unit weight of soil above phreatic level | Onverzadigd gewicht | D | 0.001 | 0.001 | ∞ |
| M03 | sat | [kN/m3] | Unit weight of soil below phreatic level | Verzadigd gewicht | D | 10 | 10 | ∞ |
| M05 | c' | [kN/m2] | Cohesion | Cohesie | S | NaN | 0 | ∞ |
| M06 | ’ | [˚] | Friction angle of shearing resistance | Hoek van inwendige wrijving | S | NaN | 0 | 89 |
| xx | su | [kN/m2] | Undrained shear strength | Ongedraineerde schuifsterkte | S | NaN | 0 | ∞ |
| M14 | S | [-] | Undrained shear strength ratio (normally consolidated) | Verhouding Su/Pc | S | 0.22 | 0 | ∞ |
| M15 | m | [-] | Strength increase exponent | Sterkte toename exp (m) | S | 0.8 | 0 | 1 |
| xx | POP | [kN/m2] | Pre overburden pressure | POP | D | NaN | 0 | ∞ |
| xx | Shear strength model | n.a. | Shear strength model (choice between C-Phi, Su-calculated and Su-measured) | Schuifsterkte model | n.a. | C-Phi | n.a. | n.a. |
| n.v.t. | Use POP | n.a. | Only relevant if the “Su-calculed” shear strength model is selected: TRUE = "Su-calculated with" and FALSE "Su-calculated with yield stress". | Gebruik POP | n.a. | False | n.a. | n.a. |

Filter Soil material parameters for Piping – Basis Module [5] in D-Soil Model:

(xxx) = value in the Technical Design of Piping kernel

xxx = current value in D-Soil Model

| WTI-ID (STPH-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. |
| M19 | WhitesDragCoefficient | [-] | Whites drag coefficient | Whites coefficient | D | 0.1 (0.25) | 0.1 (> 0) | 1 (∞) |
| M13 | D70 | [m] | 70% fractile of the aquifers grain size distribution | Diameter D70 | D | (None) | 1E-8 (-∞) | 1 (∞) |
| M14 | D70Mean | [m] | Reference value for Sellmeijer | xxx | D | 2.08E-4 | (-∞) | (∞) |
| M15 | DarcyPermeability | [m/s] | Permeability (Darcy) | Doorlatendheid | D | (None) | 0 (-∞) | 1000 (∞) |
| M12 | D50 | [m] | Median diameter of the aquifer’s grain size distribution | Diameter D50 | D | (None) | 1E-8 (-∞) | 1 (∞) |
| M07 | PipingLayer.AbovePhreaticLevel | [kN/m3] | Unit weight above phreatic level | Onverzadigd gewicht | D | 10 (None) | 0.001 (-∞) | 100 (∞) |
| M08 | PipingLayer.BelowPhreaticLevel | [kN/m3] | Unit weight below phreatic level | Verzadigd gewicht | D | 0.01 (None) | 0.001 (-∞) | 100 (∞) |
| xx | PipingLayer.DryUnitWeight | [kN/m3] | (Oven) Dry Unit weight | Ovendroog gewicht | D | 0.01 (None) | 0.1 (-∞) | 100 (∞) |
| M10 | GammaSubParticles | [kN/m3] | Submerged volumetric weight of sand particles | xxx | D | (16.5) | (-∞) | (∞) |
| M11 | BeddingAngle (revised) | [˚] | Bedding angle for the Sellmeijer revised model | Rolweerstandshoek | D | 18 (37) | 18 (0) | 72 (∞) |
| xx | BeddingAngle (original) | [˚] | Bedding angle for the Sellmeijer original model | xxx | D | (41) | (0) | (∞) |

Filter soil material parameters for Piping - Ringtoets in D-Soil Model:

| WTI-ID (STPH-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. |
| n.a. | n.a. | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. |
| M13 | D70 | [m] | 70% fractile of the aquifers grain size distribution | Diameter D70 | S | (None) | 1E-8 (-∞) | 1 (∞) |
| M15 | DarcyPermeability | [m/s] | Permeability (Darcy) | Doorlatendheid | S | (None) | 0 (-∞) | 1000 (∞) |
| M07 | PipingLayer.AbovePhreaticLevel | [kN/m3] | Unit weight above phreatic level | Onverzadigd gewicht | S | 10 (None) | 0.001 (-∞) | 100 (∞) |
| M08 | PipingLayer.BelowPhreaticLevel | [kN/m3] | Unit weight below phreatic level | Verzadigd gewicht | S | 0.01 (None) | 0.001 (-∞) | 100 (∞) |

Filter soil material parameters for D-Flow Slide in D-Soil Model:

| WTI-ID (zettings-vloeiing-..) | Parameter | Unit | Description | Name in D-Soil Model | Determinist/Stochast | Default value | Minimum value | Maximum value | Sub-Filter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n.a. | n.a. | n.a. | Soilname | Naam | n.a. |  | n.a. | n.a. | All sub-filters |
| n.a. | n.a. | ARGB-code | Color | Kleur | n.a. |  | n.a. | n.a. | All sub-filters |
| n.a. | Soil type | n.a. | Type | Grondtype | n.a. | Zand | n.a. | n.a. | All sub-filters |
| n.a. | n.a. | n.a. | Description | Beschrijving | n.a. |  | n.a. | n.a. | All sub-filters |
| M09 | D15 | [μm] |  | Diameter D15 | S | 0 | 30 | 2000 | Global / HMBreach |
| M08 | D50 | [μm] |  | Diameter D50 | S | 0 | 30 | 2000 | Global/Detailed HMBreach |
|  | ’ | [˚] | Friction angle of shearing resistance | Hoek van inwendige wrijving |  | 0 | 0 | 89 | HMBreach/Sliq2d |
|  | n | [-] | Porosity | Porositeit |  | 0 | 0.3 | 0.6 | HMBreach/Sliq2d |
|  | nmin | [-] | Minimum porosity | Min. porositeit |  | 0 | 0.3 | 0.6 | HMBreach/Sliq2d |
|  | nmax | [-] | Maximum porosity | Max. porositeit |  | 0 | 0.3 | 0.6 | HMBreach/Sliq2d |
|  | εvol;dm0 | [-] | Value of Eps_vol;dm at mean effective stress p0 | Epsvoldm0 |  | 0.0025 | 0.0003 | 0.03 | Sliq2d |
|  | Ks0 | [kN/m2] | Value of Ks at average stress p0' | Ks0 |  | 50000 | 10000 | 140000 | Sliq2d |
|  | s2 | [-] | Value of s at maximum contraction | s2 |  | 1.3 | 1.1 | 1.4 | Sliq2d |
|  | sand | [kN/m3] | Unit weight of grains | Gamma korrel | D | 26.5 | 20 | 30 | HMBreach/ Sliq2D |
|  | m | [-] | Parameter describing f(s), defined in equation: f(s) = A sm – B sr / (smax – s) | m |  | 2 | 1.5 | 3 | Sliq2D |
|  | u | [-] | Parameter describing the influence of p’ on Ks defined in equation: Ks = Ks0 (p' / p0')^u | u |  | 1 | 0.5 | 1.5 | Sliq2D |
|  | v | [-] | Parameter describing the influence of p'CON on εvoldm, defined in equation: εvol;dm = εvol;dm0 (pCON' / p0')^v | v |  | 1.25 | 0.5 | 1.5 | Sliq2D |
|  | r | [-] | Parameter describing f(s), defined in equation: f(s) = A sm – B sr / (smax – s) | r |  | 7 | 7 | 7 | Sliq2D |

Soil segment is an area or a line with consistent subsoil schematization

Soil profile is a cross section of the subsoil described in layers of material

To plot a *.sti on a map, coordinates of one point of the surfaceline is needed, as well as the location of this point on the surfaceline, i.e. only coordinates is not sufficient

The application can import borehole and CPT as GEF-file. The location of the borehole and CPT can be viewed in

map. If the location in RO is known, it is plotted in the right location. If the RO location is not known, it is not plotted on the map or at (60000, 500000).

The set of soilprofiles of the original segment apply also on the splitted parts.